HPE DL360 UEFI Booting

Approach Document

Version 1.0

Customer Restricted - Information that is intended for a restricted set of employees with the right / need to know principle relating to a specific Account engagement where information owned by the client but managed by Capgemini or information related to customers and their environments (both Technology and Business related) is being handled. This classification must be used for Capgemini documentation and systems where information is being shared with the client.

| Editing |  |  |  |  |
| --- | --- | --- | --- | --- |
| Verification |  |  |  |  |
| Validation |  |  |  |  |

Monitoring developments

| Version | Date | Editor | Description |
| --- | --- | --- | --- |
| V1.0 | 18-12-2024 | Capgemini Engineering Team | Secure boot for HPE DL360 server, Initial Version |

Table 1: Change history

Distribution

| Name | Function | Organization |
| --- | --- | --- |
| EXFO Engineering Team | Engineering | EXFO |
| CG Engineering Team | Engineering | Capgemini |

Table 2: Distribution list

Content

1	Introduction	4

1.1	Purpose of the document	4

1.2	Scope	4

1.3	References	4

2 Check secure Boot status	5

2.1 Installing Dependencies	5

2.2 Dependencies	6

2.3 Install sbsign and efi tools:	6

3. Write your own MOK keys	7

3.1 Signing Kernel module	7

3.2 Importing MOK keys	9

3.3 Enable secure boot	11

3.4 Verify secure Boot status	12

4 Inserting lastmile and writing fpga bit files	13

4.1Write the primary flash	14

5	Stakeholder	16

# Introduction

## Purpose of the document

The purpose of this document is to provide a comprehensive step-by-step guide to manage Secure Boot on CentOS 7, including generating MOK certificates, signing boot components, and verifying the setup. This ensures system integrity by aligning with UEFI Secure Boot requirements, enabling custom keys and maintaining secure configurations.

## Scope

It covers the process of configuring and enabling Secure Boot on CentOS 7. It details creating custom keys, signing boot files, and verifying Secure Boot status.

## References

- https://lists.centos.org/hyperkitty/list/announce@lists.centos.org/thread/2APH7YUSNGVRJD5LAXBI46BBB5AKWHZZ/

- https://vault.centos.org/7.9.2009/

- How to boot Linux using UEFI with Secure Boot ? — Get Intimate With Cyber ! 0.1 documentation

- https://access.redhat.com/solutions/10154

- https://www.linuxquestions.org/questions/linux-newbie-8/create-iso-from-centos-7-server-4175653114/#google_vignette

- https://www.linuxquestions.org/questions/linux-newbie-8/create-iso-from-centos-7-server-4175653114/#google_vignette

# 2 Check secure Boot status

After Flashing OS, switch to root user(su in CentOS 7).

1.To check the current Secure Boot status

mokutil –sb-state

2.Edit the Base URLs in CentOS-Base.repo

vi /etc/yum.repos.d/CentOS-Base.repo

Comment out all mirror URLs by adding a # at the beginning of each line containing mirrorlist.

Add the following baseurl lines under the respective sections:

Under [base]:

baseurl=http://vault.centos.org/7.9.2009/os/$basearch/

Under [updates]:

baseurl=http://vault.centos.org/7.9.2009/updates/$basearch/

Under [extras]:

baseurl=http://vault.centos.org/7.9.2009/extras/$basearch/

Save and exit the file:

Press Esc, then type: wq and hit Enter.

NOTE: Initially, Secure Boot must be under Disabled State.

## 2.1 Installing Dependencies

Get the files from below link

NOTE: If you don’t have wget command use the command “yum install wget“ to install wget.

wget http://10.16.128.175/New/Dependencies.tar.gz

wget http://10.16.128.175/New/Kernel.tar.gz

wget http://10.16.128.175/New/install-dependencies.sh

wget http://10.16.128.175/New/lastmile-486cf189.tar.gz

wget http://10.16.128.175/New/racer_0x1000_0x2412_0x0001.bit

wget http://10.16.128.175/New/racer_0x1002_0x2412_0x0001.bit

# 2.2 Dependencies

Provide executable permissions for install-dependencies.sh file.

chmod +x install-dependencies.sh

Run the script file to install essential tools

./install-dependencies.sh

## 2.3 Install sbsign and efi tools:

Install sbsigntools:

  - cd Dependencies/secu-os/sbsigntools

  - ./autogen.sh

  - ./configure

  - make check

  - make install

  - make install check

Install efitools:

  - cd ../efitools

  - make

  - make install

## 3. Write your own MOK keys

To generate your own MOK certificates, use the following command:

openssl req -new -x509 -newkey rsa:2048 -keyout MOK.priv -outform DER -out MOK.der -nodes -days 3650 -subj "/CN=Custom MOK Key/”

MOK.der and MOK.priv will be created.

## 3.1 Verifying and Resetting the MOK Database

Check If MOK keys are already present in MOK database using following command.

mokutil –list-enrolled

If MOK keys already exists in MOK database. Erase them using following command.

mokutil—reset

Reboot

Follow MOK management prompts to reset MOK keys.

## 3.2 Importing New MOK keys

1.Run following command to import the keys:

mokutil - -import /path/to/MOK.der

It prompts the password Twice, Provide same password both times.

2.Reboot the system and follow on-screen instructions to enroll the MOK keys and provide the password you have created during MOK key importing

## 3.3 Enable secure boot

- Reboot the system and press F9 during the boot process.

- Navigate to the following menu:

- BIOS/Platform Configuration > Server Security > Secure Boot Settings.

- Attempt to enable Secure Boot by selecting Enabled.

## 3.4 Verify Secure Boot status

To confirm Secure Boot is Enabled, Run below command in Centos 7 Terminal.

mokutil --sb-state

# 4 Inserting lastmile and writing fpga bit files

1. Build and load the lastmile driver:

cd lastmile/driver

make

./load_driver.sh

2.Build the rawcardtool utility:

cd ../rawcardtool

make

3.Verify that everything was successful.

./rawcardtool –status

4. You should see similar output to this:

./rawcardtool --flash ../../racer_0x1000_0x2412_0x0001.bit

At this point, the server needs to be rebooted in order for the FPGA card to work. Unfortunately, that means redoing steps 1 to 6 under server setup above. This is required because the factory PCI BAR size is 128KB but the new FPGA load has a BAR size of 250MB. Reboot the server by writing into the terminal:

.sudo reboot

Redo steps 1 to 6 under server setup above in order to load the driver once again.

Verify that the FPGA write was successful:

cd rawcardtool

./rawcardtool –status

Note that the FPGA Type, FeatureSet and revision correspond to the name of the flash file. If you see the same output as before writing the flash, STOP! That means that the card is not properly encrypted, and it loaded the failsafe instead of the primary flash because of that. To continue, the card must be properly encrypted and the rawcardtool status must show the output above. Once that is done, rawcardtool --status will output the correct text above.

Write the failsafe flash ONLY IF the primary flash was successfully verified in the previous step

./rawcardtool --flashsafe ../../racer_0x1002_0x2412_0x0001.bit

Reload the FPGA from the flash we just wrote:

cd ..

./reload_from_flash.sh failsafe

Verify that it was successful.

cd rawcardtool

./rawcardtool –status

NOTE: All the above steps need to be performed as a root user.

# Stakeholder

| Name | Function | Title |
| --- | --- | --- |
| Kesavan,Nivetha | Project Manager | Project Manager |
| Vijayan Bharathi | Delivery Manager | Director |
| Suresh Kalidasan | Architect | Architect |
| Navyasree | Software Developer | Engineer |
| Jhansi Vaddipalli | Software Developer | Engineer |

About Capgemini Engineering

Capgemini Engineering combines, under one brand, a unique set of strengths from across the Capgemini Group: the world leading engineering and R&D services of Altran – acquired by Capgemini in 2020 - and Capgemini’s digital manufacturing expertise. With broad industry knowledge and cutting-edge technologies in digital and software, Capgemini Engineering supports the convergence of the physical and digital worlds. Combined with the capabilities of the rest of the Group, it helps clients to accelerate their journey towards Intelligent Industry. Capgemini Engineering has more than 52,000 engineer and scientist team members in over 30 countries across sectors including aeronautics, automotive, railways, communications, energy, life sciences, semiconductors, software & internet, space & defence, and consumer products.

Capgemini Engineering is an integral part of the Capgemini Group, a global leader in partnering with companies to transform and manage their business by harnessing the power of technology. The Group is guided every day by its purpose of unleashing human energy through technology for an inclusive and sustainable future. It is a responsible and diverse organization of 270,000 team members in nearly 50 countries. With its strong 50-year heritage and deep industry expertise, Capgemini is trusted by its clients to address the entire breadth of their business needs, from strategy and design to operations, fueled by the fast evolving and innovative world of cloud, data, AI, connectivity, software, digital engineering and platforms. The Group reported in 2020 global revenues of €16 billion.

Get the Future You Want | www.capgemini.com

This document contains information that may be privileged 
or confidential and is the property of the Capgemini Group.

Copyright © 2022 Capgemini. All rights reserved.